# ODHS I2C with MPU6050 using STM32L432KCU6

# Problem statement

Read from the who am I register of the gyroscope(mpu6050) using I2C protocol using STM32L432KC

# theory of i2c read and write

in i2c generally sequential write and read is supported (as it is in the case of mpu6050).

## To write to a specific register:-

- start bit

- address of slave

- R/W bit (=0 to write)

- Get Ack from slave

- Send 1st register address to be written to

- Send data (the data is written to the said register)

- If any following data is sent, it is sequentially written onto register+i th register

- Stop bit

## To read data from a register:-

- Start bit

- Address of slave

- R/W bit =0

- Get ack from slave

- Send 1st register address to be read

- Ack from slav

- Then resend start bit

- Address of the slave followed by R/W bit =1 (to read mode)

- Slave send ack and sends the data on that register

- If master sends back ACK then the slave sends the data in register +1 . this keeps happening till when master keeps sending ack

- To stop the conversation, master send a NAK followed by stop bit

# MPU6050

The MPU-60X0 is an integrated 6-axis MotionTracking device that combines a 3-axis gyroscope, 3-axis accelerometer, and a Digital Motion Processor™ (DMP) and a dedicated I2C sensor bus.

Electrical specifications:-

Part 6.3 of MPU6050 Datasheet

- VDD can vary from 2.375V to 3.46V and is Thus tied to the 3V output of the STM32F4.. board

## I2C specifications:-

### Operating Frequency

In fast mode the MPU can do upto 400kHz and in standard mode it can do upto 100kHz

- For this project standard mode is used at a frequency of 100000 Hz i.e. 100kHz

### Address of MPU6050

The address of MPU6050 is :-

When AD0=0 address= 1101000 (using this)

When  AD0=1 address= 1101001

- The AD0 pin is connected to GND to make it 0 and thus the Address of the MPU6050 = (0b110100<<1 +0)

### Register to be accessed

Register 117- Who AM I is to be accessed

# Setting up the project

- Open a new STM32 project on STM32cubeIDE and select the STM32L432KCU6 processor.

- Connect the RCC Clock

- According to the datasheet of STM32L4

PA9= I2C1 SCL

PA10 =I2C1 SDA

Thus the pins are configured as such

- Under connectivity>I2C1 > mode is turned from disabled to i2c

All the parameters are left as default

i.e. i2c speed mode = standard mode

i2c speed frequency = 100kHz

- In the file /Drivers/STM32L4xx_HAL_Driver/Inc/stm32l4xx_hal_i2c.h  , there are 3 different types of IO operation functions available for I2C

- Blocking mode: Polling  - In this mode, the CPU waits for the communication and cant be used for anyother task (hence blocking)

- Non blocking mode : Interrupt mode – works on interrupt basis

- Non blocking : DMA mode- DMA is a hardware process that can handle data transfers without processor intervention. Thus, events can happen behind the scene without needing to interrupt the processor

- In this case,polling mode is used.

/* IO operation functions  ****************************************************/

/******* Blocking mode: Polling */

HAL_StatusTypeDef HAL_I2C_Master_Transmit(I2C_HandleTypeDef *hi2c, uint16_t DevAddress, uint8_t *pData,

uint16_t Size, uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_Master_Receive(I2C_HandleTypeDef *hi2c, uint16_t DevAddress, uint8_t *pData,

uint16_t Size, uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_Slave_Transmit(I2C_HandleTypeDef *hi2c, uint8_t *pData, uint16_t Size,

uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_Slave_Receive(I2C_HandleTypeDef *hi2c, uint8_t *pData, uint16_t Size,

uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_Mem_Write(I2C_HandleTypeDef *hi2c, uint16_t DevAddress, uint16_t MemAddress,

uint16_t MemAddSize, uint8_t *pData, uint16_t Size, uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_Mem_Read(I2C_HandleTypeDef *hi2c, uint16_t DevAddress, uint16_t MemAddress,

uint16_t MemAddSize, uint8_t *pData, uint16_t Size, uint32_t Timeout);

HAL_StatusTypeDef HAL_I2C_IsDeviceReady(I2C_HandleTypeDef *hi2c, uint16_t DevAddress, uint32_t Trials,

uint32_t Timeout);

here for normal i2c communications first four functions (i.e HAL_I2C_Master_Transmit to HAL_I2C_Slave_Receive are used)

where as to read registers/memory HAL_I2C_Mem_Write and HAL_I2C_Mem_Read is used

HAL_I2C_IsDeviceReady is used to check whether the device is ready or not.

# Code

In the main.c file

- Define the device address and register address

- /* USER CODE BEGIN PD */

- #define MPU6050_address (0x68<<1)+0 //device address with AD0 to GND

- #define who_am_i_reg_add 0x75//register address

- /* USER CODE END PD */

- Main function

- int main(void)

- {

- /* USER CODE BEGIN 1 */

- HAL_StatusTypeDef ret; //to store the return of the function

- uint8_t rData; // data buffer to store the data read

- /* USER CODE END 1 */

- /* MCU Configuration--------------------------------------------------------*/

- /* Reset of all peripherals, Initializes the Flash interface and the Systick. */

- HAL_Init();

- /* USER CODE BEGIN Init */

- /* USER CODE END Init */

- /* Configure the system clock */

- SystemClock_Config();

- /* USER CODE BEGIN SysInit */

- /* USER CODE END SysInit */

- /* Initialize all configured peripherals */

- MX_GPIO_Init();

- MX_I2C1_Init();

- /* USER CODE BEGIN 2 */

- ret=HAL_I2C_Mem_Read(&hi2c1,MPU6050_address,who_am_i_reg_add, 1,&rData, 1, 1000);

- /* USER CODE END 2 */

- /* Infinite loop */

- /* USER CODE BEGIN WHILE */

- while (1)

- {

- /* USER CODE END WHILE */

- /* USER CODE BEGIN 3 */

- }

- /* USER CODE END 3 */

- }

Function used:-

- Upon building the project, console:-

- 20:01:25 **** Incremental Build of configuration Debug for project i2c_STM32L4_MPU6050 ****

- make -j8 all

- arm-none-eabi-gcc "../Core/Src/main.c" -mcpu=cortex-m4 -std=gnu11 -g3 -DDEBUG -DUSE_HAL_DRIVER -DSTM32L432xx -c -I../Core/Inc -I../Drivers/STM32L4xx_HAL_Driver/Inc -I../Drivers/STM32L4xx_HAL_Driver/Inc/Legacy -I../Drivers/CMSIS/Device/ST/STM32L4xx/Include -I../Drivers/CMSIS/Include -O0 -ffunction-sections -fdata-sections -Wall -fstack-usage -fcyclomatic-complexity -MMD -MP -MF"Core/Src/main.d" -MT"Core/Src/main.o" --specs=nano.specs -mfpu=fpv4-sp-d16 -mfloat-abi=hard -mthumb -o "Core/Src/main.o"

- ../Core/Src/main.c: In function 'main':

- ../Core/Src/main.c:70:27: warning: variable 'ret' set but not used [-Wunused-but-set-variable]

- 70 |         HAL_StatusTypeDef ret; //to store the return of the function

- |                           ^~~

- arm-none-eabi-gcc -o "i2c_STM32L4_MPU6050.elf" @"objects.list"   -mcpu=cortex-m4 -T"D:\parikshit\ODHS\STM32CubeIDE\STM32\i2c_STM32L4_MPU6050\STM32L432KCUX_FLASH.ld" --specs=nosys.specs -Wl,-Map="i2c_STM32L4_MPU6050.map" -Wl,--gc-sections -static --specs=nano.specs -mfpu=fpv4-sp-d16 -mfloat-abi=hard -mthumb -Wl,--start-group -lc -lm -Wl,--end-group

- Finished building target: i2c_STM32L4_MPU6050.elf

- arm-none-eabi-size  i2c_STM32L4_MPU6050.elf

- arm-none-eabi-objdump -h -S i2c_STM32L4_MPU6050.elf  > "i2c_STM32L4_MPU6050.list"

- text	   data	    bss	    dec	    hex	filename

- 10052	     20	   1652	  11724	   2dcc	i2c_STM32L4_MPU6050.elf

- Finished building: default.size.stdout

- Finished building: i2c_STM32L4_MPU6050.list

- 20:01:26 Build Finished. 0 errors, 1 warnings. (took 1s.248ms)

# Debugging and output

- MPU6050 is connected as follows :-

| PIN name (on STM32) | STM32 Pin | Connected to (pin on MPU6050) |
| --- | --- | --- |
| D1 | PA9 | SCL |
| D0 | PA10 | SDA |
| 3V | - | Vcc |
| GND | -- | GND |
| GND | - | AD0 |

- STM32cubeIDE is put into debugging mode, the code is debugged

## OUTPUT:-

# Errors:-

When using RCC LSE clock , the HAL_I2C_Mem_Read function returns the following.

Solution:- Thus, HSE clock is used

# References:-

https://www.youtube.com/playlist?list=PLmXXQ1iFwiyKKURU4wAeGT_d1HsIe1YCY

MPU6050 datasheet:-

https://invensense.tdk.com/wp-content/uploads/2015/02/MPU-6000-Datasheet1.pdf

MPU6050 register map:-

https://invensense.tdk.com/wp-content/uploads/2015/02/MPU-6000-Register-Map1.pdf

STM32L462KCU6 Datasheet:-

https://www.st.com/en/microcontrollers-microprocessors/stm32l432kc.html#documentation

HAL Documentation:-

https://www.st.com/resource/en/user_manual/um1725-description-of-stm32f4-hal-and-lowlayer-drivers-stmicroelectronics.pdf